RRSG Seminar 2025

Rory Schram

Hello to all of those of you in person, and online, and welcome to the seminar session today. I hope everyone is doing well.

Today, I have three main goals, much like the last time I presented, which consist of the following. Firstly, I would like to explain again, exactly what the heck I am trying to achieve, and explain the reasoning behind why such a project could be useful. Secondly, I will go over what I have done since the last time we had a little check in. Thirdly, I will explain what my next steps will be.

So, starting with point 1, what the hell am I trying to achieve? Well, at its bare bones basics, I am trying to design a RADAR system, that can manipulate a communication protocol to be able to classify targets within a simple RADAR frame of interest. Lets look at how a very bare bones pulse radar might operate in the context of my project.

Firstly, a baseband signal must be generated. The choice of signal can vary dramatically depending on the use case. But to keep it simple, lets look at the use of a simple chirp signal.

A chirp is a signal who’s frequency varies with time. This frequency doesn’t have to change linearly with time, but often it does. I have created one for demonstration purposes here. For my purposes, I work with real and imaginary information. The following chirp has a bandwidth of 2MHz, as it spans the frequency space from -1 MHz to 1MHz. The signal is 25 000 samples long. This signal gets a little bit messy, but I have zoomed in on the right hand side to show more of the signal.

Represented here is the time domain portion of the transmitted signal. And here is the frequency domain representation of the signal.

Once we have this signal, the system will upmix a this baseband signal onto a carrier signal and transmit this signal at a scene.

This signal will propagate through the air, until it hits a target, in which case the target will reflect the radio waves.

This reflection will propagate back to the radar setup, in which case a receive antenna will receive the said reflection and downmix the received signal to baseband. This received signal will be very messy and may look something like this…

From here, the radar system can process the data. For an extremely simple radar such as this pulse RADAR, this processing could start with removing any DC offset in data. Then the originally transmitted data can be correlated with the received data. This step is the pulse compression step so common to simple RADAR systems. This is the result of performing a correlation between the transmit signal, and a theoretical return signal which I created, which contains the original signal, delayed by 2000 samples. Since this theoretical system is sampling at 25 MHz, a 2000 sample delay being added in equates to a target distance of around 12 km. Obviously, a system like this in the real world would not have a reflection signal that is this powerful. But for demonstration purposes, I like to show a strong return signal.

If we zoom in once again, we can see two peaks clearly. The first big peak is antenna crosstalk. That is, the signal that immediately gets detected at the RADAR system’s receive antenna as transmitting begins. I have done this simulation based around the workings of my system, and my system has crosstalk, therefore this simulation will have crosstalk. You can see that the second peak obtained by the correlation is placed exactly at 2000 samples after the crosstalk, which is expected.

Now, if we wanted to make this RADAR way better, we could broadcast multiple chirps instead of just one, effectively creating a pulsED RADAR. We could then perform pulse integration to get the best results possible for each capture. Then we could create a range-time intensity map, which we could infer velocity information from. This is however out of scope, and for my purposes I have decided to stay with the simple range maps.

Now, this bare bones pulse radar is great, but there is a flaw, we have no idea what we are looking at. So how do we go about solving this problem? Well, I don’t know what other people do in this regard, but I can tell you about how I am trying to do it.

Well, for my system, I utilize an orthogonal frequency division multiplexed signal as the baseband signal for my RADAR. So, instead of the chirp function we saw earlier, my baseband signal that I now use looks something like this. Last time we talked, I had just about gotten to this step. I had essentially created a signal that could be used as a communication signal aswell as a RADAR signal, except that it performed seriously badly.

Since then, I completely redesigned the construction of my OFDM signal. To do this, I had to create a toolchain from scratch. This toolchain works in the following way.

To generate an OFDM based signal, one must work in the frequency domain. So essentially, we create an array of IQ samples. Then we have to create the broad signal structure. For my use case I settled on the following signal parameters. The total carriers in the signal is 32768. This number is chosen to make FFTs and IFFTs efficient, although not very necessary in this experimental space. My signal has 24 000 active subcarriers. Of those 24 000 active sub carriers, 4000 are pilot symbols. These pilot symbols are symbols placed into the signal that have a known value. They don’t all have the same value to avoid repetition, however they all have a known value. These pilot symbols are extremely important as they give us the ability to create a channel estimation of from our transmissions, and these transmissions are what we are aiming to use to classify objects in frame.

After creating the broad structure of the signal, we are left with 20 000 4 QAM data slots, giving us the ability to fill 40 000 bits worth of data in our signal. I create 40 000 random bits and use that in the signal.

After creating this signal, we now have a complete, OFDM based signal, that can 1) be used in a pulse compression radar to determine ranges of objects, 2) can be used to transmit and receive actual data in the form of bits, and 3) can have a channel estimation extracted from it, which can be used for classifying targets.

I have also done a complete redesign of the receive side of things…

So how do I use this to try and classify objects that I am looking at?....

Embedded in this signal that I transmit, are pilot carriers. These

This pretty much brings us up to speed with where I was up to last time.

Last time, I had built a prototype system that was able to transmit

So what would this look like in the real world, well